| ## Power Distribution #### Fuse to Components Index | 22-8 |  |
| --- | --- | --- |

Under-dash Fuse/Relay Box

| Fuse Number | Amps | Wire Colour | Component(s) or Circuit(s) Protected |
| --- | --- | --- | --- |
| 1 | 15A | BLK/WHT | Ignition coils |
| 2 | - | - | Not used |
| 3 | 20A | - | Not used |
| 4 | 10A | BLK/ORN | Alternator, ELD unit, EVAP purge control solenoid valve, Primary HO2S, Secondary HO2S, Vehicle speed sensor, Reverse lockout solenoid, CMP sensor |
| 5 | 7.5A | Fuse/relay box socket | Multiplex control unit |
| 6 | 7.5A | YEL/GRN | Headlight adjuster switch, Headlight adjuster unit, Power window master switch |
| 7 | 20A | - | Not used |
| 8 | 7.5A | Fuse/relay box socket | Optional connector V, Multiplex control unit |
| 9 | 10A | GRN | Rear window wiper motor, Rear window washer motor, Rear window intermittent wiper control unit |
| 10 | 7.5A | Fuse/relay box socket | Multiplex control unit |
|  |  | YEL | Back-up lights, EPS control unit, Gauge assembly, Keyless receiver unit, Security control unit |
| 11 | 7.5A | BLK/ORN | ABS modulator-control unit |
| 12 | - | - | Not used |
| 13 | 10A | PNK | SRS unit |
| 14 | 10A | Fuse/relay box socket | Optional connector V |
|  |  | BLK/YEL | Blower motor relay, Heater control panel, Power mirror actuator, Power mirror defogger, Radiator fan relay, Rear window defogger relay, Recirculation control motor |
| 15 | 30A | - | Not used |
| 16 | 20A | - | Not used |
| 17 | 15A | BLK/YEL | Inertia switch, SRS unit |
|  |  | YEL/BLK | ECM, Fuel pump |
| 18 | 15A | YEL/GRN | Cigarette lighter |
| 19 | 7.5A | YEL/BLK | Turn signal/hazard relay |
| 20 | 20A | Fuse/relay box socket | Multiplex control unit |
|  |  | GRN/BLK | Windshield wiper motor, Windshield washer motor |
| 21 | - | - | Not used |
| 22 | 20A | GRN/BLK | Passenger's power window motor |
| 23 | 20A | GRN/WHT | Driver's power window motor |
| 24 | - | - | Not used |
| 25 | - | - | Not used |