| ## Design Specifications #### (cont'd) | 2-35 |  |
| --- | --- | --- |

| Item | Measurement | Qualification | Specification |
| --- | --- | --- | --- |
| ELECTRICAL RATINGS | Battery |  | 12 V-36 AH/5 hours |
|  |  |  | 12 V - 45 AH/20 hours |
|  |  |  | 12 V - 38 AH/5 hours |
|  |  |  | 12 V - 47 AH/20 hours |
|  | Starter |  | 12 V - 0.8 kW, 1.0 kW, 1.2 kW |
|  | Alternator |  | 12 V - 70 A |
|  | Fuses | Under-hood fuse/relay box | 80 A, 60 A, 40 A, 30 A, 20 A, 15 A, 7.5 A |
|  |  | Under-dash fuse/relay box | 20 A, 15 A, 10 A, 7.5 A |
|  | Light bulbs | Headlight high beam | 12 V - 60 W |
|  |  | Headlight low beam | 12 V - 55 W |
|  |  | Front turn signal/Front side marker lights | 12 V - 21 W/5 W |
|  |  | Front position lights | 12 V - 5 W/3 CP (candlepower) |
|  |  | Front parking lights | 12 V - 5 W |
|  |  | Side turn signal lights | 12 V - 21 W |
|  |  | Rear turn signal lights | 12 V - 21 W |
|  |  | Brake/Taillights/Rear side marker light | 12 V - 21W/5 W |
|  |  | Rear fog light | 12 V - 21 W |
|  |  | Inner taillight | 12 V - 5 W |
|  |  | High mount brake light | 12 V - 21 W |
|  |  | Back-up light | 12 V - 21 W |
|  |  | License plate light | 12 V -3 CP |
|  |  | Ceiling light | 12 V - 8 W |
|  |  | Trunk light (4-door) | 12 V - 5 W |
|  |  | Luggage area light (5-door) | 12 V - 5 W |
|  |  | Individual map light | 12 V - 8 W |
|  |  | Spotlights | 14 V - 8 W |
|  |  | Gauge light | 14 V - 1.4 W |
|  |  | Indicator light | 12 V - 1.12 W |